Vessel Monitoring and Maintenance Checklist

| Item | Planned maintenance and service items | Frequency of inspection/work |
| --- | --- | --- |
| # STRUCTURE |  |  |
| Hull / deck / superstructure (external) | Inspect hull after slipping / dry-docking. Assess: seams, planking, laminate, plating, welds. Carry out maintenance and repair work as found necessary. Inspection and maintenance carried out on total external area of hull. |  |
| Underwater valves and openings including fittings above waterline | Disassemble valves. Check for wear and corrosion, stem glands. Ensure handle in satisfactory condition. Clean, grease and reassemble or replace. Assess bolts if flanged and skin fitting. |  |
| Hull / deck / bulkheads / superstructure (internal) | Thorough internal inspection of hull framing, bulkheads, deck and deck framing, bulwarks. Thorough inspection of superstructure, windows and closing arrangements. |  |
| Tanks and spaces | General inspection of tank and framing and closing arrangements and venting. |  |
| Load bearing deck fittings | Inspect fitting and bolting arrangements and structure within the vicinity of the fitting. |  |
| Hull protection | Underwater areas painted at each slipping and topsides if necessary. Anodes replaced or additional fitted as necessary. |  |
| # WATER TIGHT INTEGRITY |  |  |
| Weather tight doors and seals | Repairs as necessary, plus frequently monitored. |  |
| Hatch covers / securing devices | Repairs as necessary, plus frequently monitored. |  |
| Hatch coamings | Repairs as necessary. |  |
| Ventilators, dampeners / air pipes | Repairs as necessary, plus frequently monitored. |  |
| Portholes and deadlights | Resealed if reported leaking at hull joint. Gaskets checked ensure dogging bolts are free, the deadlights seal and glass is not damaged. Repair or replace if damaged which renders the ship unsafe. |  |
| Freeing ports / scuppers | Maintain free of obstructions. |  |
| # GENERAL ARRANGEMENT & MISCELLANEOUS ITEMS |  |  |
| Signs limiting passenger access | Ensure still legible and appropriate |  |
| Upper deck rails | Stanchions secure? Rails and / or wires in good condition, no broken strands |  |
| Handrails throughout ships | Ensure secure and remain adequate for purpose. |  |
| Ladders, stairs | Secure, ensure tread is not worn smooth |  |
| Deck surfaces | Ensure non skid finish and in good condition |  |
| Windlass, anchors, cables and warps | Thorough inspection, replace worn parts. |  |
| Masts / rigging |  |  |
| Structural fire protection | Ensure no degradation of material or attachment. |  |
| Escape arrangements | Ensure unobstructed and workable. |  |
| Passenger seating and berths | Secure and in good condition. |  |
| Toilets, showers and hand basins | Operational, in good condition and hygienic. |  |
| LP gas system | Certificate current and good condition. |  |
| Sullage / sewage tanks | Not leaking. |  |
| Waste pumps / pipe work | Operational, in good condition. |  |
| Sails | Operational, in good condition. |  |
| # MACHINERY |  |  |
| Main and auxiliary engines | Servicing periods. Performance of engine constantly monitored. Fuel supply filters constantly monitored, see operations manual. Lube oil sampling every……………. | Serviced in accordance with engine makers recommendations. |
| Overhaul periods gear box | Serviced in accordance with makers recommendations. Serviced to maintain fluid levels. |  |
| Rudders, stocks, pintles, bearings etc | Inspect for wear, corrosion, mechanical damage, locking arrangements. Carry out repairs and renewal as found necessary. |  |
| Stern gear | Stern gear, all components from gear box to propeller nut. |  |
| Main steering gear | Performance constantly monitored. Inspection includes all components of steering system from wheel to rudderstock assessing wear and corrosion. Oil leaks in hydraulics systems and condition of hydraulic lines. Repairs and adjustments carried out as found necessary. Ensure stops are effective. | Thorough inspection carried out every ………… ……………………...……... |
| Emergency steering gear | Test conducted and manual tiller confirmed on board in good condition and can be put in place for use. |  |
| Fuel tanks | Tanks separate to hull structure. Tank plating inspected every………. ......................................................... Areas of tanks against the hull to be inspected. Note: As far as can be inspected without disturbing the tank but must be able to form a confident opinion as to the condition of the tank. Fuel tanks inspected internally (this inspection may not be necessary for a tank with a capacity of less than 800 litres). Maintenance work carried out as found necessary. Tank securing arrangements. Tanks forming part of the hull-structure inspected and maintained as per hull. |  |
| Pipe work and fittings including tank fittings and transfer pumps | Fuel delivery and return system inspected every…………………. ……………………………………… Any leaks or other defects are repaired immediately and clean-up procedures carried out. Fuel tanks isolating arrangements tested as per operational manual. Any repairs or adjustments, found necessary, carried out immediately. Ensure isolating arrangements are operating efficiently. |  |
| Pressure vessels / safety valves | Maintain as per Australian Standard 3788. |  |
| Air compressor (diving ) | Maintain fluid levels, ensure filters are clean, monitor performance. |  |
| Remote controls - engine, gear box, other machinery | Continuously monitor and lubricate. |  |
| Refrigeration (includes safety valve) | Service and repairs by specialist. Following items frequently monitored by crew: Pipe condition Electrical connection HP cut-out Oil levels LP cut-out Refrigerant levels Belt drives Fans, pumps, etc. | Inspection tests and services carried out by specialist-trained* person every …………………… *this person must have a licence – www.arctik.org |
| Air conditioning | Service and repairs by specialist*. Following items frequently monitored by crew: Pipe condition Electrical connection HP cut-out Oil levels LP cut-out Refrigerant levels Belt drives Fans, pumps, etc. | *this person must have a licence – www.arctik.org |
| Bilge / fire pumps and manual bilge pumping | Tested in accordance with operational manual. Repairs and adjustments carried out if poor performance is observed. Condition of drive belts and control arrangements continuously monitored. |  |
| Bilge alarms | Repairs and adjustments as found necessary. |  |
| Pipe work | Check for corrosion, cracks, deteriorated flexible sections, damaged or missing hose clips, strainers. Condition of any pipe supports. | Thorough inspection conducted every ………… |
| # ELECTRICAL |  |  |
| Low Voltage (240 / 415 volts) system | Visually inspect the electrical installation, including; plugs, socket, extension leads, and portable equipment. Check for mechanical damage, corrosion, watertight integrity of fittings in exposed locations and exposed wiring. The complete installation should be inspected by a Queensland Transport accredited electrical surveyor or licensed electrical mechanic. Certificate of Compliance for Survey, Form 3104, issued by an accredited electrical surveyor or Electrical Installation / Inspection Statement, Form 1588, issued by an electrical mechanic. | Weekly Annually Every 4 years or when repairs or modifications are performed |
| Extra Low Voltage (12 / 24 volt) battery supplied system | Batteries: check the general condition, tightness of terminals and electrolyte levels of wet cells Inspect the complete electrical installation. Check for cleanliness, mechanical damage, corrosion, watertight integrity of fittings in exposed locations and exposed wiring or terminals. | Weekly Annually |
| Emergency electrical gear | Inspect as for Low and Extra low voltage as appropriate. Test operation change over devices and alternate supplies. | Weekly |
| SAFETY EQUIPMENT (Including stowage, marking and launching arrangements |  |  |
| Inflatable life rafts and release | Serviced as required by legislation. |  |
| Rigid life rafts | Serviced as required by legislation. |  |
| Rescue boat / lifeboat | Regular inspection |  |
| Lifeboat equipment | Regular inspection. |  |
| Lifebuoys, lights and lines | Continuously monitor if appliances are accessible to passengers. Conduct thorough inspection of casing, lines, reflective tape, markings legible. | Check every |
| Lifejackets (PFD) - list type | Inspect overall condition of jacket including straps, reflective tape. Where buoyant material has hardened or flattened, jacket to be discarded. Are they readily accessible? |  |
| Lifejacket (PFD) lights - other approved lights | Check batteries. If lithium or similar, check manufacturers expiry date. If dry cell or alkaline type, renew annually and record details. |  |
| Distress signals & EPIRB | Test battery regularly and replace battery and flares before expiry. | Check every ……………... |
| Emergency lighting | Test | Weekly |
| # MISCELLANEOUS EQUIPMENT |  |  |
| Anchors | Inspect condition of all anchors and cables on board every…………….. Assess wear, corrosion and any deformations due to applied loads. Repair or replace as necessary. |  |
| Cables | Check end for end chain cables to ensure evenness of wear. Ensure bitter end is attached to ship. |  |
| Hawsers and warps (mooring lines) | Condition monitored for wear and UV degradation. |  |
| Medical stores | Check itemised list and product expiry date. |  |
| Magnetic compass-swung? | Maker’s recommendations. Should be swung every 3 years or if modifications are carried out. |  |
| Navigational aids/charts/ radar | Check connections, monitor performance. Charts and notices to mariners - ensure current. |  |
| Gangway / pilot ladder, safety net | Modifications to be carried out to gangway if circumstances of ship's operations change, which may render the gangway inefficient or unsafe. Continuously monitor condition. |  |
| Navigational lights | Checked as per operational manual, ensure lens maintain colour, arc of visibility and position. |  |
| NUC lights, day shapes | Ensure they are undamaged and can be displayed. |  |
| Radio/s | Radio checks used to monitor performance refer Operations Manual. Radio technician may be engaged to service appliance, transmitters, aerial and earthing. |  |
| LPG equipment type | System inspected by licensed gas installer as required by legislation (Gas Act). Bottle stowage and condition of system continuously monitored. Any repairs necessary or modifications to be carried out by a licensed installer and a certificate obtained and placed in the maintenance record. Condition of appliances. |  |
| # FIRE FIGHTING EQUIPMENT |  |  |
| Fire detection/ smothering system, description of system | Should be surveyed / serviced in accordance with Section 11 of the USL Code. Extinguishing container shall be tested and inspected in accordance with Section 11 of the USL Code. Installation / service Certificate? |  |
| Non-portable and portable fire extinguishers | Serviced in accordance with manufacturer's recommendations. Pressure gauges checked during pre-departure checks. Dry powder units shaken regularly to ensure powder has not compacted. |  |
| Fire bucket, blanket, axe | Service certificate |  |
| Fireman's outfit and breathing apparatus |  |  |
| Fire main, water service pipes and hydrants | Tested in accordance with operational manual. Repairs and adjustments carried out if poor performance is observed. Condition of drive belts and control arrangements continuously monitored. |  |
| Hoses, nozzles and couplings | Monitor during fire drills to ensure serviceability. |  |
| # SPECIALISED MACHINERY |  |  |
| Cranes / davits |  |  |
| Winches |  |  |
| Pile frames |  |  |
| Dredge plant |  |  |
| Fish processing |  |  |
| Trawling equipment |  |  |
| # STABILITY |  |  |
| Load line draft marks | Ensure marks are conspicuous. Repaint if necessary, including deck line mark. |  |
| Stability information | Carried on board? Refer section dealing with hull for closing arrangements. Ensure deck-freeing ports are not obstructed (drainage). Check that any special conditions imposed by the stability book are complied with, for example, ballast, 'No Go' signage for upper decks. |  |